Software Design and Implementation

WCDE-00087-03
Revision 100621

# csa propagation software development

# Software Design, Implementation and Testing

Ilia V. Baranov and David Effa

The design phase aims to develop a solution for requirements identified in the previous stage. In order to do that, the core C++ code presented in Revisiting Spacetrack Report #3 [1] has been used as a baseline for creating an IVB SGP4 propagator. The flow chart for this code is shown in Figure 1. . This code presents most of the required algorithms to accomplish propagation, however it was meant to be an academic demonstration of the principle, not an operational program. Therefore, several modifications were required to render the program fully usable. These changes fall into two categories: feature addition and functionality modification.

Figure 1. Revised Program Code Flowchart.

Feature addition to the program entailed adding functions and outputs not present in the original code. The first addition was the enabling of variable time steps. This allows the user to specify the time interval between adjacent data lines. Another addition was the division of output formats into four types: Full Data, Visible Data, Pass List and Solar. Full Data format outputs all available data at each time step from start time to stop time. Visible Data is similar to Full Data except it will only output data when the satellite is visible from the station in question. Pass List format outputs only the rough information pertaining to multiple orbits over the requested time period. Lastly, Solar Format outputs information pertaining to the sun’s position in the sky over the given time period. This information is used to avoid pointing the antenna directly into the sun.

Another major addition was the creation of a sub method to project XYZ coordinates onto a two dimensional earth map. This was used to create a real-time display of the satellite’s position on the map, in order to see upcoming passes visually. A screenshot of the program’s two-dimensional projection is presented in Figure 2.

Figure 2: 2D projection of two passes

The program also required functionality modification to make it compatible with the TT&C system. These modifications included better algorithms and buffer usage. First, the Pass List method used a recursive algorithm in order to increase execution speed. The method first performs a rough estimation of pass times, and then calls itself with the results of the previous recursion. This approach yields precise results without wasting computation time on periods when the satellite is not visible. Another modification was the use of buffers and files when outputting data. The program writes all data for an entire data line to a buffer, and only then writes to a file. This combination significantly increased writing efficiency and doubled execution speed.

During the implementation phase, which is the longest phase of the software development life cycle, the IVB SGP4 software was developed from design phase deliverables, . Here, the code is developed from core C++ source code. It is also produced to run on a hardware system, using specific technologies for specific platforms.

IVB SGP4 Testing and Results Verification

During testing, the implementation is tested against the requirements to make sure that the product is actually solving the needs addressed and gathered during the requirements phase.  Unit tests and system/acceptance tests are done during this phase.  Unit tests act on a specific component of the system, while system tests act on the system as a whole. While IVB SGP4 program code was mathematically correct, a few programming errors were discovered during unit testing. A significant error encountered during IVB SGP4 implementation was related to a timestamp issue. The code would calculate values at odd intervals, leading to irregular time references for a specific coordinate. This was found to be caused by a rounding error encountered when using doubles for relative time calculations. These calculations involve calculating the Julian Date and the Sidereal Time, both of which entail a multistep process.. The error introduced in each calculation caused the timestamp to accumulate garbage on the order of microseconds with each step. This caused an accumulating offset on the timestamp for each data set, eventually rendering the data completely useless. This effect is presented in the Table 1 below.

Table 1: Timestamp Offset in the original code

| Requested Time in minutes from TLE Epoch | Returned Time |
| --- | --- |
| -1440.00000000 | 0:0:0.000000 |
| -1430.00000000 | 0:10:0.000004 |
| -1420.00000000 | 0:20:0.000009 |
| -1410.00000000 | 0:30:0.000013 |
| -1400.00000000 | 0:40:0.000018 |
| -1390.00000000 | 0:49:59.999982 |
| -1380.00000000 | 0:59:59.999987 |

This problem was remedied by forcing the program to round the timestamp value every integer multiple of the time step. Thus, the trailing digits were erased, and then used to re-calculate the most recent coordinates. This effectively erased the increasing error introduced by the irregular timestamps. Another timestamp problem was found in the starting and stopping time. Since the requested time was input in minutes, but then recalculated in Julian Date seconds, a slight rounding error was incurred. The program would not provide data starting and stopping exactly when requested. This difference was on the order of approximately one second; however this was a significant difference in operational terms. The solution devised was to re-calculate the desired Julian date starting and stopping time using the original input data, but subtracting the difference between the desired date and the calculated date. This shifted the calculated time to exactly match the requested time.

A limitation imposed on the program in Pass List Mode was the omission of any pass that starts or stops outside the range of time specified by the user. This was due to the architecture of the C++ and Labview™ interface. This limitation will likely remain in place due to the difficulty of providing an accurate out-of-bounds orbital prediction.

Another limitation was that IVB SGP4, when compared to output from STK™ Version 8, was shown to have an oscillating residual with an increasing error over time. This error is under 0.07Km over 4 days for the XYZ coordinates. This means Full Data, Visible Data, and Solar Data from this program should not be utilized when generated for a period exceeding ±5 days from the TLE Epoch Time. Pass List Data should similarly not be used when exceeding 20 days. This limitation may be improved upon by putting more work into the mathematical development; however, there was no need to implement such a long-term prediction at that time.

The challenges presented in integrating the format and improving on the data were numerous. However, the final program provided data in the proper format and was easily integrated into the existing system. This leads to the conclusion that IVB SGP4 adheres to the design specifications sufficiently to be used in the operational system.

System Test Methodology

The only failsafe way to check output data from IVB SGP4 was to compare the results calculated to other, professionally used, programs as well as real world recorded data. This was accomplished through the implementation of several Labview™ programs designed to graph and compare the data. The raw XYZ coordinates were compared between IVB SGP4 and STK™, another professional orbital propagation program, using AGI™. The antenna azimuth and elevation data was also compared between real life data, IVB SGP4 and STK™. The residual between the IVB SGP4 and the other two methods where compared to identify any systematic error or bias. Full data, Visible data and Solar data were compared in this fashion, while Pass list was compared against the scheduling software available in-house. A comparison between the original SGP4 code and IVB SGP4 is also provided.

Results verification

The first verification was the comparison of XYZ coordinates provided by IVB SGP4 and STK™. This test was carried out by graphing both data sets as well as calculating the residual between them. One such comparison is presented in Figure 5.

Figure 5: Y-axis Coordinate comparison

As can be seen above, the scale is too large to allow any difference to be measured between the data sets. For this reason a residual graph was produced, indicated in Figure 6.

Figure 6: Y Coordinate Residual

It can be seen above that the residual between STK™ and IVB is minimal, ranging from -0.023 to -0.013 km. Although this residual is minimal, it seems to indicate systematic bias. The bias is demonstrated when taking a propagation over the space of a few days. This propagation is shown in Figure 7.

Figure 7: X coordinate propagation over 4 days

While such an increasing oscillation for a residual is not favorable, there are a few reasons why IVB SGP4 output is still valid. First, while STK™ is assumed accurate, the precision of entered data is not very high. This indicates that the residual may be attributed to different methods of calculation. In addition, even the worst deviation is only -0.09km, which is so small that resulting antenna angles are not affected. Lastly, use of IVB SGP4 under normal circumstances does not call for such extended periods of time. Therefore, this issue will not impact normal use.

As has been shown in this section, IVB SGP4 clearly produces reasonably accurate propagation results and has the same characteristics as STK™, which is a professional program. This, along with more extensive testing performed during the validation, leads to the conclusion that IVB SGP4 is sufficiently accurate and consistent for use within the operational environment over normal lengths of time

Conclusions and Recommendations

After performing a range of tests using the new program, IVB SGP4 was deemed suitable for use in the TT&C system. This conclusion is substantiated by the testing results outlined above, which served to validate the design of the program against the requirements specified in the first phase of the development process. Ultimately, program output was shown to closely resemble STK™ and real world data, which suggests it is accurate enough for orbital prediction applications used by TT&C division.

The first recommendation encompasses the changes that must be made to the current system in order to utilize the new system. The Labview™ interface programs should be re-written to implement more parallel processing in order to take advantage of the multi-instantiable nature of the code. In addition, the data input to the propagator has changed and will need updating as well.

IVB SGP4 incorporates many more features than the previous propagation solution. By having all of these features in the same program, station automation becomes possible. Interface programs must be written to utilize the raw data provided by IVB SGP4 in order to drive station systems such as the main antenna, converters, and so on. The code must also be consistently verified against newer algorithms and odd-orbit satellites. Doing so will ensure that the program remains up to date and will catch any

errors.

To accomplish all these tasks, human resources along with sufficient technical support and time must be invested to further this project. If this project is not continued, then the resources and expertise currently invested will go to waste. In addition, the original problem will persist, increasing costs along the way. It is for this reason that the final recommendation is for the immediate investment of the above mentioned resources into further development of this project into an operational system.

References

[1] 	Vallado, David A., Paul Crawford, Richard Hujsak, and Kelso, T. S., "Revisiting Spacetrack Report #3," AIAA/AAS Astrodynamics Specialist Conference, Keystone, CO, Aug 2006.

[2]  Ilia V. Baranov and David Effa, “CSA propagation software development”, WCDE 00087-01, Waterloo Cases in Design Engineering, July 2010

[3] Ilia V. Baranov, “SGP4 Propagation Program Design and Validation”, 1A Work Term Report, Department of Electrical and Software Engineering, University of Waterloo, Waterloo, ON Canada, April 13, 2009

[4]    http://celestrak.com/publications/AIAA/2006-6753/AIAA-2006-6753.pdf.