Oil Pump Dissection:

Internal Gear Pump

AKA

“Outside Gear/Inside Gear”

Jordan Lee

ME100

How does an internal gear pump work?

- Anatomy

- Stages of Function

- Advantages and Disadvantages

- Conclusion

- Questions

Anatomy

- Four key components

  - Pump housing

  - Idler – interior spur gear

  - Rotor – exterior spur gear

  - Crescent

Stages of Function

- Oil enters the gap between the idler and rotor

- Oil travels with rotation of the idler and rotor, dividing at the crescent

  - Seal is created between the suction and discharge ports through division

- Gear gaps are constant, providing constant flow volume control

- As teeth completely mesh and seal, oil is forced out the discharge port

Picture taken from:

http://cast.csufresno.edu/agedweb/agmech/graphics/hydraulics7.gif

Advantages and Disadvantages

| Advantages | Disadvantages |
| --- | --- |
| - Simplistic design | - Requires moderate speeds |
| - Positive suction | - Medium pressure limitations |
| - Ideal use for high viscosity substances | - One of the bearings must run within the substance pumped |

Conclusion

Through this presentation, one should:

- Recognize the components of;

- Understand the functions of;

- Be familiar with the advantages and disadvantages of;

An internal gear pump.

Questions

Resources

All information and more can be found at:

Internal Gear Pumps:

http://www.pumpschool.com/principles/internal.htm