Sliding Vane Pump

Manufacturing Components:

- Externals (head, casing, bracket) - Cast iron, ductile iron, steel, and stainless steel.

- Vane, Pushrods - Viton®, Ryton®, PPS, carbon, Teflon®

- Liner & End Plates - Cast iron, ductile iron, steel, carbon, and stainless steel.

- Bearing Cap - Cast iron, ductile iron, steel, and stainless steel.

- Shaft Seal - Component mechanical seals, industry-standard cartridge mechanical seals.

Sliding Vane Pump Advantages

- Medium capacity

- Medium speed

- Thin liquids

- Handles various liquids (not just oils)

- Can run dry for short periods

- Can have one seal or stuffing box

- Develops good vacuum (can even run for while with small air penetration)

Sliding Vane Pump Disadvantages

- Can have two stuffing boxes

- Complex housing

- Not suitable for high pressures

- Not suitable for high viscosity

- Not good with abrasives

Applications:

Various Versions of Sliding Vane Pumps are found in such devices:

- Aviation Service - Fuel Transfer, Deicing

- Auto Industry - Fuels, Lubes, Refrigeration Coolants

- Barge Unloading

- Bulk Transfer of LPG and NH3

- LPG Cylinder Filling

- Ethanol/Alcohol Refining